Upcoming low temperatures and increased risk of fires. It is expected that electricity consumption will increase due to the connection of a larger number of heating appliances to the electrical network, their use around the clock at a higher heating mode. All this creates a prerequisite for overloading the electrical installation.

To prevent fires, follow the basic rules: A fire in a heating appliance most often occurs due to improper use: placing flammable objects (clothes, furniture) nearby, not extinguishing the solid fuel stove to the end, leaving smoldering coals, or poorly maintained and uninsulated chimneys/stoves, which leads to the spread of fire to neighboring rooms or the release of deadly carbon monoxide. To avoid, it is key to follow safety rules - distance from combustible materials, proper cleaning, fuel storage and checking the condition of the device.

Main causes of fires from heating appliances:

Proximity to combustible materials: Clothing, furniture, curtains that are dried or placed too close to the stove, fireplace or radiator. Improper disposal of slag/ash: Uncooled coals can spontaneously ignite if disposed of near flammable waste. Incomplete combustion (smoldering): Carbon monoxide (CO), a colorless, odorless gas that can lead to suffocation, is released. Gaps and cracks: In the case of masonry stoves and chimneys, through which heat passes to neighboring rooms. Improper fuel storage: Coal or wood stored in moisture can self-ignite.

How to prevent a fire: Keep a distance: Keep all flammable objects at least 1-2 meters away from the heater. Clean regularly: Ashtrays and firebox should be cleaned once everything has cooled completely. Do not leave unattended: Do not leave working stoves or fireplaces unattended, especially for long periods of time, or while sleeping, if they are not fully secured. Check the chimney: Ensure good insulation and cleaning of the chimney, without cracks. Store fuels properly: In a dry place, away from living quarters. Consider a CO detector: Install a carbon monoxide detector near appliances to be alerted to incomplete combustion.